Set up a connection through a router by XPORT

The supplied patch cable is a crossover cable, which means you can connect it directly to the PC / laptop. But also by a hub, switch or router.

Configure Virtual COM port via Port Redirector

Note: Test the XPORT first with the supplied patch cable directly or through a hub, switch or router. The green LED should be on and on the PC a network connection must be must be displayed

install Cpr32DL_4.2.0.0_Web.exe (on the CD under XPORT_Remote_Control).
If .net is not installed start dotnetfx.exe
(On the CD under \ XPORT_Remote_Control \. Net \) first.
Restart the computer!
The computer should have the fixed IP address 192.168.1.2, Sub-Net 255.255.255.0 (change in System-Settings, Network, select Adapter,select Properties, select TCP/IP, select Properties and change. 
Start CPR manager
After calling "Search Devices" the XPORT device should appear at the bottom.
Then "Add Com Port" (top left icon)
Select the COM port (any)
Then Input at service the host IP address of the XPORT (192.168.1.11) and TCP port 10001
If the access is via DSL then specify here the host address off dyndns.org 
Then press "Save Settings" button.
By “Test Com x” it can be tested by Open / Close the XPORT.
The program can now be closed. The driver is started automatically by Windows. The program must not be started every time.
If, however, you access through the Internet and DSL and you restart Windows you must first call the CPR-Manager and enter the IP address you get from dyndns.org.


Now start the Software MP_Remote, select the according COM port and Stop and start again
The data of metamorphosis should be displayed
Enjoy!

Also a download of new software via the bootloader is possible! (also select the according COM port)


That’s the mask of the CPR Manager. Enter First "Search", then Add COM port, IP-data host 192.168.1.11 and port 10001, then Save.

If the XPORT operate over the Internet, you can continue "XPORT operate over the Internet under

Links for XPORT: 
www.lantronix.com

XPORT should operate over the Internet via DSL

If the XPORT should operate over the Internet via DSL, the following is required:
Sign in to www.dyndns.org
The DSL router has a dynamic IP address. This changes depending on the settings every 24 hours or each time you dial.

To know this IP address is known, we need an account with a DynDNS provider
Therefore we have to go for example to 
http://www.dyndns.org/account/create.html
and sign in

After activating the account, we place first our host (the name by which we will be found). If you have not done it please login with the
User data in http://www.dyndns.org log (on the website top right). To create a new host, we must switch to the "Services" and then "Add Host".
In the following form, we can now enter our chosen host name and link it to a domain.
 

Host Name: wttest.dyndns.org
Where wttest is our chosen dyndns.org host name and a registered domain is from our suppliers.

IP-Adress Please leave it as it is!

Here is the IP that you were currently assigned by your ISP.

enable Wildcards 

DynDNS Update
Now, the router must report its current IP to dyndns

Setting in the router
select with the Internet Explorer, the address 192.168.1.1 (check the manual which address belongs to the special router)
Here, the automatic login can be configured:
 
If this is not possible at the router so there is still the possibility to install a software on the PC to send the IP address of the router to dyndns. Otherwise skip the following section

Infos about that is here:
http://www.portforward.com/routers.htm

assigned fixed IP addresses internally to the router

That  the XPORT can be clearly identified, I recommend to have internal fixed IP addresse assigned to the XPORT
 
In this example, the third line is the XPORT. The MAC-address of the XPORT is on the housing of the XPORT

Note: How to get to the MAC address of the PC's?
In Windows, Start, Run, cmd, IP Config / all:
 
The MAC address is in the example 00-0B-5D-2D-32-9B

Unblock ports on the router
For the XPORT, the following ports have to be unblocked:
10001, 30718, 43282, 43283

In the router firewall select Add Rule:
 
At available service define the ports
For list of destinations-Adress in our example should be 192.168.1.36
And then save;

In this overview, it looks like this (usually 1 to 3):
 


Forward Ports to the appropriate PC
Now you have to forwarded the Port to the appropriate to the appropriate PC.
This goes with NAT (Network Address Translation), also known as port forwarding on the router:
 

For IP address enter the IP-Adress of the XPORT (eg: 192.168.1.36), which we have assigned to the desired computer (at static DHCP)

Thus, the settings at the Router are completed

Connection problems?
If there are Connecting-problems, select in the firewall the according port.
Does a ping to the address works?
Are the computers in the same IP group (first two IP addresses match)? This is via the Internet, of course not necessary.


Additional Links:
http://www.portforward.com/
http://www.portforward.com/routers.htm
http://www.wintotal.de/Artikel/dyndns/dyndns.php
http://www.heise.de/ct/05/10/096/
http://www.tim-bormann.de/index.php?section=184

For inquiries look Newsgroups in the Internet

Contact
Stefan Schranner
Tel: +49 (0) 931 72 353
Mobile: +49 (0) 152 54690448
Email: StefanSchranner@yahoo.de

Wuerzburg, 28.11.2007