# PC-Software MP Remote-Program

This Software is for Loading of the measured values on the PC and fully Operating the Metamorphosis with the PC

Overview Functions by the PC-Software MP Remote-Program

- Select the according serial COM-Port. If you use a USB-Adapter the according COM-Port has to be selected.

- Select the according Software and you have access to all documents and Plans by the Menu „Software and „Doku“.

- Monitoring and fully control of the Metamorphosis (changing of all Parameter etc). This can be realized also by modem and Mobile-Phone. You can press the keys with the mouse or with the key Alt + Number.

- Loading of the stored Measurement-Values in a Text-File pressing the key „Load Data to PC”

- Open this file with Measurement-values by the Menu “Data, View Uploaded Data”

- Loading of a new Program; Start the „Bootloader“-Software from the Menu. In there you have to select the serial COM-Port and the according Program.

- If you have a Modem connected then you can dial with „Telephone“ the according number. In this Menu you can also disconnect the connection.

- The connection can be established by Mobile phone as well.

- Activation of the Sending of Data to Logview or any other Program by the Menu Data, „Data, Send Data for Logging (LogView)“. After activation Stop the MP Remote-Program and Start Logview. And select as Device “Metamorphose_Solar_e”. Refer to Document “Logview-Info” for more Information.

- All Parameters which are Setup in Menu 8 can be uploaded to a Text-File by the Menu “Data, Get Parameters”.

- They can be modified in the Text-File. Select the File by Menu Data, change Parameters. Then they can be loaded again to the Metamorphosis by Menu “Data, Send Parameter”. By that you have more Information on each Parameter and a backup of your Data is possible.

- All Parameters which are Setup in the Universal-Module can be uploaded to a Text-File by the Menu “Data, Get Data Universal-Module”.

- They can be modified in the Text-File. Select the File by Menu Data, change Parameters. Then they can be loaded again to the Metamorphosis by Menu “Data, Send Data Universal-Module”. By that you have more Information on each Parameter and a backup of your Data is also possible.

- By the Button „Display all Data“ you can enlarge the Display and more Data are shown. But for the Input of Data it is better to press the Button “Stop Display for Input of Data” temporarily.

- By the key “Simulation” a Simulation-Value for testing can be Setup

- In Menu “Data, Edit Input/Output-List” the Name of all Inputs and Outputs can be Setup for Documentation. For the Alarm-System (later the other Units as well) it can be downloaded to the Metamorphosis by Menu “Data, Send Input/Output-List”. With this Text the SMS is generated and also in Menu 4 you have the according information if you select the Input.

# Installation

On the CD you find the Software „MP Remote-Program“. How to install?

- In order to control the control via the PC, if not already done, the software "MP Remote-Program" must be installed from the CD to the hard disk. Simply call \ PC_Remote_Control \ install.bat.

- To register the OCX, the following must be done:

Navigate to the C: \ Metamor directory through the commands

For 32-bit operating systems:

There click with the right mouse click "as administrator" register32.bat

For 64-bit operating systems:

There with the right mouse click "as administrator" register64.bat click

- • For Vista, Windows 7 and higher do the following:

- • Select properties of the shortcut on the desktop

- • Select compatibility

- • Select as administrator and close the window.

- • Start application

- • Then he brings the query UAC rights. Then confirm this

- Now connect the metamorphosis with the cable you get to  the serial interface of the PC

- Then call the program C:\Metamor\”MP_Remote.exe”

- After the start selected the COM-Port. Begin with COM1 and if it doesn't go select COM2

Loading of the Logged Data to the PC

- Now press the button”Upload”; the data will now be loaded and ”End” will be displayed when finished.

- Now you can look at the data with the function ”Data” and "View uploaded data" you can write comments, print it and import them in Microsoft Word or Excel for example (import to Excel: File, open, Typ "all files", then go to folder C:\Meta\data.txt, then Data is separated by Blank)

Contact

Stefan Schranner

Frau-Holle-Weg 28

97084 Würzburg

Tel: +49 (0)931 72353

Mobil: +49 (0) 152 54690448

Internet: www.alternative-technologie.de

Email: StefanSchranner@yahoo.de